TABLE 1

DVRPC HIGH PRIORITY ROUTE 70 PROJECTS

FOR SAFETY, MOBILITY, AND CONGESTION

1.  For the Garden State Park Redevelopment, which will add 2,662 a.m. peak hour trips; 4,490 p.m. peak hour trips; and 3,944 Saturday peak hour trips:

- Identify transit opportunities that provide alternatives to auto trips (i.e. neighborhood shuttle buses, connections between transit hubs and shopping destinations.

- Identify important pedestrian crossings and development improvements which promote safety and accessibility.

- Implement traffic calming measures and traffic management techniques which facilitate safe and efficient flow of vehicles.

2.  For the new Cooper Landing Road left stacking lane:

- Add a traffic signal at the median opening for Cooper Landing Road to operate in coordination with the signal at the Edison/Georgia intersection with Route 70 located at milepost 3.  According to the DVRPC, “permissive left turns are inherently unsafe, with safety decreasing dramatically when motorists cross multiple lanes, in this case two lanes.”

3.  For the intersection of Route 70 and Kingston Road/ West Gate Drive:

- Optimize the signal and implement/ optimize signal coordination to greatly improve traffic flow and reduce congestion.

- Increase the capacity of left turn movements from side streets by creating dedicated dual left turn lanes at intersections with greatest demand for such turns.

- Construct more pedestrian crossing opportunities, especially near bus stops, that are well marked and signal controlled to benefit transit dependent and all pedestrians.  There should be at least one pedestrian crossing opportunity every [.3] miles, including crossings that link retail and commercial centers on either side of Route 70.

- Provide more left turn opportunities beyond those currently under construction by constructing additional directional mid-block openings with left turn lanes.

- Add a third signalized intersection at Renaldo Terrace and the entrance to the Barclay Farms Shopping Center.  DVRPC report stated that this “is a logical location” for a third signalized intersection” as it is “nearly equidistant from the adjacent two intersections (Kingston and Covered Bridge), and it would provide signalized left turn access to and from the shopping center.  It would provide an additional signalized opportunity to turn left from the north side of Route 70 easing the congestion at Kingston Drive by distributing the burden.”

4.  Covered Bridge/ Frontage Road

- To better accommodate left turn movements from Frontage Road (368 during the a.m. peak hour) add a split-phase signal configuration to allow each minor street approach to clear uninhibited by the opposing minor street movement.

- Actuate signal so that the movement with the greatest number of queuing vehicles gets more time.

- Alter geometry to configure signal to provide a protected left turn phase allowing the unimpeded flow of left turns from both approaches simultaneously.

- Add a second left turn lane from Frontage Road southbound.

5.  Greentree Road/Springdale/ Marlkress Road Jug Handle.

- Reword signs and add arrows to state more clearly that the Marlkress Road jug handle is the only way to access Green Tree and Springdale Road northbound from Route 70 eastbound.

DVRPC “NJ Route 70 Corridor Study,” October 2005,  p. 72

Same, p. 73.  This improvement is specifically listed as having safety benefits.

Same, p. 77

Same, p. 77

Same, p. 77

Same, p. 78

Same, p. 78

Same

Same, p.79

Same, p. 80

Same, p. 81  This improvement is specifically listed as having safety benefits.